# <School Name>

<School Address>

# <Building name>

<Targeted Seismic Assessment/Detailed Seismic Assessment>

Date: enter date here
Report Revision: enter here
Consultant Project Number: enter here
Prepared By: Consultants name here
For the New Zealand School Property Agency

# Document Control

Document Prepared By:

<Full Legal Company Name>

<Company Address Physical>

<Company Address Postal>

Telephone: <Company Phone number>

Email:	<Company email>

Web: <Company Website>

### Revision History

| Revision No. | Prepared by | Description | Date |
| --- | --- | --- | --- |
| A | <name> | <Draft for review> | <xx/xx/xxxx> |
| B | <name> | <Final> | <xx/xx/xxxx> |

### Document Acceptance

| Who | Name | Signed | Date |
| --- | --- | --- | --- |
| Site visit by: | <name> <position> |  | <xx/xx/xxxx> |
| Prepared by: | <name> <position> |  | <xx/xx/xxxx> |
| Reviewed by: CPEng No: <xxxxxxx> | <name> <position> |  | <xx/xx/xxxx> |
| Approved by: CPEng No: <xxxxxxx> | <name> <position> |  | <xx/xx/xxxx> |

Contents

Document Control	1

Revision History	1

Document Acceptance	1

Executive Summary	4

Assessment Results	4

What do these Results Mean for <School>?	4

Next Steps	5

1.	Introduction	7

1.1	Background	7

1.2	Purpose	7

1.3	Scope	7

2.	Site Description	9

2.1	Geotechnical Site Conditions	9

3.	Building Description	11

3.1	Building Overview	11

3.2	Site Walk-Over and Building Condition	11

3.3	Gravity Load Resisting System	12

3.4	Lateral Load Resisting Systems	13

3.4.1	Transverse Direction	13

3.4.2	Longitudinal Direction	13

3.4.3	Foundations	13

3.5	Secondary Structural and Non-Structural Elements	13

4.	Detailed Seismic Assessment Results	14

4.1	Basis of Assessment Summary	14

4.2	Summary of Results	14

4.3	Critical Structural Weakness	15

4.4	Structural Weakness	15

4.5	Severe Structural Weaknesses	15

4.6	Building Drifts	16

4.7	Stairs	16

4.8	Secondary Structural and Non-structural Elements	16

5.	Seismic Strengthening Considerations	18

6.	Conclusion and Next Steps	19

6.1	Conclusion	19

6.2	Next Steps	19

7.	Explanatory Statement	20

Appendix A - Assessment Summary Table	21

Appendix B - Sources of Information	26

B.1	Existing Documentation	26

B.2	Geotechnical Information	26

B.3	Site Observations and Investigations	26

Appendix C - Basis of Assessment	28

C.1	Analysis Methodology	28

C.2	Material Properties	28

C.3	Key Input Parameters	29

C.3.1	Seismic Loads	29

C.3.2	Dead and Live Loads	30

C.4	Assumptions	30

Appendix D - Existing Documentation	31

Appendix E - Detailed Seismic Assessment Calculations and Analysis Results	32

# Executive Summary

<Consultant X> has been engaged by the New Zealand School Property Agency (NZSPA) to complete a <Targeted Seismic Assessment (TSA)/Detailed Seismic Assessment (DSA)> of <building name> at <school name>. This <TSA/DSA> evaluated the expected performance of the building during a seismic event and identified any building vulnerabilities.

<short description of the building including age (if known) and the LFRS>.

## Assessment Results

The seismic assessment provides an earthquake rating for the building expressed as a percentage of new building standard (%NBS). The overall rating is based on the lowest scoring element, referred to as the critical structural weakness. Elements scoring below 100%NBS are building vulnerabilities, known as structural weaknesses. Table 1 summarises the assessment results.

Table 1: Summary of the Key Seismic Assessment Results

| Item | Results |
| --- | --- |
| Earthquake rating | <XX%NBS (ILY)> |
| Lowest scoring element (critical structural weakness) | <CSW> |
| Elements scoring less than 34%NBS | <xx> <xx> |
| Elements scoring less than 100%NBS (structural weakness) | <xx> <xx> |

## What do these Results Mean for <School>?

<building name> has element(s) rated below 34%NBS (ILX).  This means the building has structural vulnerabilities that could pose a risk during an earthquake. Strengthening considerations have been provided that would improve the buildings seismic performance. <either> As there are elements that score below 34%NBS, this building requires strengthening in order to meet NZSPA’s policy objective of achieving at least 34%NBS.

<Or>

These should be considered at the time of other building upgrades (e.g. refurbishment, weathertightness works) and as informed by NZSPA.

The %NBS rating is not a direct measure of safety but an indicator of how the building is expected to perform in an earthquake compared to current Building Code. Decisions about occupancy and upgrades are made based on a combination of technical and non-technical factors, rather than just %NBS. NZSPA has developed the School Seismic Policy and Seismic Risk Procedures, which defines the process for decision making around continued occupancy. These processes align with Seismic Risk Guidance developed by MBIE.

## Next Steps

Carry out the <additional> intrusive investigations identified in this report.

NZSPA to review the draft assessment report and confirm requirements for seismic strengthening. This may include a seismic risk evaluation to inform occupancy decisions.

Develop strengthening works, if required.

Table 2: Seismic Assessment Summary Table

|  | Information Category | Building-specific Information |
| --- | --- | --- |
| Building Information | School | <School name> |
|  | Block no (K2) | <K2 block number or number off site plan> |
|  | Block name/description | <e.g. main hall, admin block or classroom block> |
| Building Description | Year of design/construction | Design circa: Construction circa: |
|  | Building typology | Choose an item. |
|  | Known standard design | Choose an item. |
|  | Number of levels (excluding mezzanines or part basements) |  |
|  | Primary structure | <describe the primary lateral load resisting system> |
|  | Secondary structure (incl. heavy non-structural elements) | <outline any relevant secondary structure> |
|  | Previous seismic strengthening or alterations, and date | <outline previous structural strengthening or alterations and date> |
|  | Current usage | <outline the use of the building e.g. classroom block, administration building, gum, hall etc> |
| Assessment Information | Assessment type | Choose an item. |
|  | Date building inspected | <xx/xx/xxxx> |
|  | NBS rating (Importance Level) | <XX%NBS (ILX)> |
|  | Significant building vulnerabilities and mode of failure |  |
|  | Geotechnical considerations | <add any other known/relevant geotechnical aspects such as the site topography that may influence likely building performance> |
|  | Other considerations | <e.g. note any significant degradation of the building observed> |

Introduction

Background

<Consultant X> has been engaged by NZSPA to complete a <Targeted Seismic Assessment (TSA)/Detailed Seismic Assessment (DSA)> of <school name, building name>, located at <address>.

Commentary: 
Provide any background information including previous assessments.

Purpose

The purpose of this engineering assessment is to evaluate the expected performance of the building during an earthquake.

<follow with either>

This DSA involved a comprehensive quantitative assessment of the whole structure to determining the overall earthquake rating and to identify any building vulnerabilities.

<or>

This TSA involved a quantitative assessment of the selected elements listed below to determine the earthquake rating of these elements and to identify any building vulnerabilities.

<xx>

Refer to NZSPA’s School Seismic Policy and Seismic Risk Procedures for further information on the types of assessments, relevant legislation, and commonly used terminology.

Scope

The scope of this assessment includes the following:

Review available documentation such as calculations, drawings, specifications and geotechnical information. Refer to Appendix B for a list of existing documentation used for this assessment.

Inspection of the building to observe its structural systems for the purposes of this assessment, confirm alignment with existing documentation and visually assess its current condition.

Undertake a detailed analysis in accordance with The Seismic Assessment of Existing Buildings – Technical Guidelines for Engineering Assessments (Version 1, July 2017) and relevant amendments (C1, C2, C3, C5, C7).

Apply NZSPA Structural and Geotechnical Requirements (Version 3.0, October 2020) throughout the assessment.

Use engineering judgement to identify significant life safety hazards, referencing the JC 25-01 report (April 2025) from the Joint Committee for Seismic Assessment and Retrofit of Exiting Buildings.

Provide recommendations for a range of seismic improvements that could be undertaken to improve the buildings seismic performance.

Prepare a <Targeted Seismic Assessment/Detailed Seismic Assessment> report using NZSPA’s reporting template.

Liaise with NZSPA and the Engineering Advisory Group (EAG). This includes the following hold points:

Completion of the initial documentation review, to discuss identified significant life safety hazards (SLSH).

Completion of the draft report, to discuss SLSH and concept strengthening options.

If there is a safety hazard identified during the assessment that has the potential to immediately cause an injury, then appropriate steps to isolate the hazard, or isolate people from the hazard needs to be undertaken. NZSPA is to be contacted immediately.

Site Description

2.1	Building Location

<building name> at <school name> is located at <address>. The building is located at the

Figure 1: Site Plan

Insert image snapshot of the site showing building identity and location clearly. Reference source of aerial imagery (e.g. LINZ, Web Map Services) and orientate photo with north aligned to the top of the page.

<north/south/east/west/centre> of the site as shown in Figure 1.

Geotechnical Site Conditions

The geotechnical report prepared by <Consultant Y> titled <’title’> dated <xx month xxxx> has been used for this assessment. A full list of sources is found in Appendix B. Table 3 provides a summary of geotechnical conditions and parameters for the site.

Table 3: Summary of Geotechnical Parameters for the Seismic Assessment

| Geotechnical Parameter | Description | Comment |
| --- | --- | --- |
| Ground conditions | <e.g. Medium dense silt deposits on dense sandy gravels> | <e.g. based of 1 bore hole at the site and 2 bore holes withing 50m> |
| Assessment category | <structurally dominated, interactive, geotechnically dominated> | <e.g. This assessment is governed by slope stability> |
| Subsoil Class | <D> |  |
| Ultimate bearing capacity | <XXX kPa> | <e.g. Strength reduction factor = 1.0 for assessment> |
| Liquefaction effect including settlements | <e.g. Susceptible to liquefaction. 45mm settlement at ULS including 35mm per 6 m of differential settlement. Low risk of lateral spread> |  |
| Potential geohazards | <e.g. soil/foundation deformations, land instability causing loss of support for the building, fault rupture near the building etc> |  |

Commentary:

Provide an overview of the geotechnical conditions and parameters including the geotechnical hazards. The parameters in the table are not exhaustive.

Building Description

Building Overview

Commentary:

Provide a succinct summary of the building, including the number of storeys (excluding mezzanine or part basements), building construction type, year of design/construction etc. Note any structural alterations, including the approximate year(s) of upgrade if known, and provide details of the modifications. Provide photos to support this section

It is recommended that the gross floor area and year of design/construction are validated and do not solely rely on NZSPA data.

Figure 2: <Northern> Elevation

Insert half page photo - elevation

Site Walk-Over and Building Condition

A <non-intrusive/intrusive>, visual site walk-over was completed by a structural engineer on <XX/XX/XXXX>. The purpose of the inspection was to verify the structural layout, where possible, against available structural drawing information.

Commentary:

Provide a statement on the alignment between the available existing information and the site observations. Note any discrepancies. List any intrusive investigations and what was found. This can be a summary with further detail and photos provided in Appendix B.

Example text:

It was found that the building generally aligned with the available existing documentation with the following exceptions:

<observation>

<observation>

Commentary:

Provide a summary of the building condition including any observations made during the site walk-over. This can be a summary with further detail and photos provided in Appendix B.

Example text:

The structure was found to be in <good/fair/poor/very poor> condition. The following observations on the building condition were made

<observation>

<observation>

Site photographs and additional details from the site walk-over are provided in Appendix B.

Gravity Load Resisting System

Commentary:

Provide a succinct summary of the gravity load resisting system. Provide annotated drawings, model images, and/or site photos to support the description

Lateral Load Resisting Systems

Commentary:

Provide a succinct summary of the lateral load resisting system. Provide annotated drawings, model images, and/or site photos to support the description. The subsections below are suggestions.

Transverse Direction

Longitudinal Direction

Foundations

Secondary Structural and Non-Structural Elements

Commentary:

Provide a succinct summary of the secondary structural and non-structural elements. Provide annotated drawings, model images, and/or site photos to support the description

Detailed Seismic Assessment Results

Basis of Assessment Summary

Commentary:

Provide a summary of the basis of assessment. Detail to be provided in Appendix C

Further details on the analysis method, material properties, key input parameters and assumptions are provided in Appendix C.

Summary of Results

Table 4 summarises the %NBS scores for the various elements in the building resulting from the detailed assessment, with relevant commentary on results.

Table 4: Summary of Results

| Element | %NBS (ILX) | Commentary |
| --- | --- | --- |
| <Element including location> | XX%NBS |  |
| <e.g. Precast concrete cladding panels on gridline A> | <e.g. 15%NBS> | <e.g. Out-of-plane failure due to the inadequate anchorage of the precast concrete panel connections. Panels may fall away from the building onto areas outside the building.> |

The assessment finds that the building achieves an overall earthquake rating of XX% NBS (ILY).

Commentary:

For each element, provide the mode of failure in the commentary box. If the element is below 34%NBS, provide a consequence statement and if it is considered a significant life safety hazard (SLSH). Refer to the Joint Committee for Seismic Assessment and Retrofit of Existing Buildings Report JC 25-01 Applying Engineering Judgement in Determining When a Significant Life Safety Hazard Occurs, dated April 2025. This commentary will help inform the seismic risk evaluation and occupancy decisions.

If the element description is not clear, use annotated drawings, model images, and/or site photos to support descriptions.

Critical Structural Weakness

The critical structural weakness (CSW) is the lowest scoring element assessed and governs the overall earthquake rating of the building. For <building name>, <CSW> is the critical structural weakness. <discuss>

Commentary:

State and discuss the critical structural weakness. Provide further commentary on the mode of failure and consequence of failure. Describe any redundancy that may be available from secondary load paths.

Structural Weakness

A structural weakness (SW) is any aspect of the building structure and/or the foundation soil that scores less than 100%NBS. For <building name>, the SWs are:

<xx>

<discuss>

Commentary:

State and discuss the structural weaknesses. A subsection could be provided for each structural weakness if there are multiple.

Severe Structural Weaknesses

A severe structural weakness (SSW) is a predefined structural weakness that is potentially associated with catastrophic collapse and for which the capacity may not be reliably assessed based on current knowledge.

<either>

No SSW have been identified for this building.

<or>

For <building name>, the severe structural weakness(es) identified are:

<xx>

<discuss>

Commentary:

State and discuss any severe structural weakness.

Building Drifts

Table 5 provides a summary of the building drifts.

Table 5: Summary of Building Drifts

| Level | Peak Drift (mm, %) | Commentary |
| --- | --- | --- |
| Level 1 | <xx mm, xx%> |  |
| Roof | <xx mm, xx%> |  |

Commentary:

Summarise the peak building drifts (magnitude and percentage) if there are adjacent structures or drift sensitive elements. These can be provided as an estimate and/or ranges. Provide discussion including any risks from or to adjacent buildings and commentary on seismic gaps (if any).

Stairs

Commentary:

State and discuss stair construction and supports with reference to the DBH Practice Advisory 13:  Egress Stairs.

Secondary Structural and Non-structural Elements

Table 6 provides a summary of the building drifts

Table 6: Secondary Structural and Non-structural Elements

| Element | Description | Comment |
| --- | --- | --- |
| <Cladding> |  |  |
| <Stairs> |  |  |
| <Ceilings> |  |  |
| <Services/Plant> |  |  |
| <Appendages/Signs> |  |  |
| <xx> |  |  |

Commentary:

Describe the Secondary Structural and Non-structural (SNSS) elements that have been included within this assessment, and their scores (with reference to Tables A4.1 and A4.2 of the Engineering Assessment Guidelines).

In addition, provide comment on any of the following aspects:

heavy façade elements

heavy light fittings (individual light fittings of greater than 5kg in weight that are not fixed directly to framing)

heavy heating units

other unsecured heavy fixtures or furniture or overhead items

portions of buildings that interface with other buildings

Seismic Strengthening Considerations

Commentary:

Provide recommendations for seismic improvements that could be undertaken to improve the buildings seismic performance. To support informed decision-making, a range of upgrades shall be explored and document. These options should include:

The scope, scale and staging (where appropriate) of proposed interventions.

The expected %NBS rating outcomes.

Associated costs, timelines, and impacts on building use.

Identification of vulnerabilities addressed and those remaining.

Discuss seismic strengthening considerations. Include where step changes in performance are apparent, and identify interim risk mitigation options if applicable. Use annotated drawings, model images, and/or site photos to support descriptions.

NZSPA will be looking for maximum gains for best economic efforts.

To improve the seismic strength and resilience for the building, the following improvements are recommended:

<xx>

The following additional intrusive investigations are required:

<xx>

Commentary:

Provide a list of additional intrusive investigations if required. Use annotated drawings and site photos if appropriate.

Conclusion and Next Steps

Conclusion

The building achieves an overall earthquake rating of XX%NBS when considered as an Importance Level X building. This is governed by <CSW>

<Either>

As this rating is less than the earthquake prone threshold in the Building Act of 34%NBS, this building is likely to be determined earthquake prone by <XXX> Council.  As required by the EPB Methodology, an Assessment Summary Table is provided in Appendix A.

This building requires strengthening in order to meet NZSPA’s policy objective of not being earthquake-prone.

<Or>

High level seismic strengthening considerations have been outlined. These should be considered at the time of other building upgrades (e.g. refurbishment, weathertightness works) and as informed by NZSPA.

Next Steps

The next steps for <building name> are:

Carry out the <additional> intrusive investigations identified in Section 5.

NZSPA to review the draft assessment report and confirm requirements for seismic strengthening. This may include a seismic risk evaluation to inform occupancy decisions.

Develop strengthening works, if required.

Explanatory Statement

Commentary:

Provide an explanatory statement.

Example text:

This report contains the professional opinion of XXXX as to the matters set out herein, in the light of the information available to it during preparation, using its professional judgment and acting in accordance with the standard of care and skill normally exercised by professional engineers providing similar services in similar circumstances. No other express or implied warranty is made as to the professional advice contained in this report.

We have prepared this report in accordance with the brief as provided and our terms of engagement. The information contained in this report has been prepared by XXXX at the request of its client, NZSPA, and is exclusively for its use and reliance. It is not possible to make a proper assessment of this report without a clear understanding of the terms of engagement under which it has been prepared, including the scope of the instructions and directions given to and the assumptions made by XXXX. The report will not address issues which would need to be considered for another party if that party’s particular circumstances, requirements and experience were known and, further, may make assumptions about matters of which a third party is not aware. No responsibility or liability to any third party is accepted for any loss or damage whatsoever arising out of the use of or reliance on this report by any third party.

The report is also based on information that has been provided to XXXX from other sources or by other parties. The report has been prepared strictly on the basis that the information that has been provided is accurate, complete and adequate. To the extent that any information is inaccurate, incomplete or inadequate, XXXX takes no responsibility and disclaims all liability whatsoever for any loss or damage that resulting from any conclusions based on information that has been provided to XXXX.

Assessment Summary Table

| 1. Building Information |  |
| --- | --- |
| Building Name/ Description |  |
| Street Address |  |
| Territorial Authority |  |
| No. of Storeys |  |
| Area of Typical Floor (approx.) |  |
| Year of Design (approx.) |  |
| NZ Standards designed to |  |
| Structural System including Foundations |  |
| Does the building comprise a shared structural form or shares structural elements with any other adjacent titles? |  |
| Key features of ground profile and identified geohazards |  |
| Previous strengthening and/ or significant alteration |  |
| Heritage Issues/ Status |  |
| Other Relevant Information |  |

| 2. Assessment Information |  |
| --- | --- |
| Consulting Practice |  |
| CPEng Responsible, including: Name CPEng number A statement of suitable skills and experience in the seismic assessment of existing buildings |  |
| Documentation reviewed, including: date/ version of drawings/ calculations previous seismic assessments |  |
| Geotechnical Report(s) |  |
| Date(s) Building Inspected and extent of inspection |  |
| Description of any structural testing undertaken and results summary |  |
| Previous Assessment Reports |  |
| Other Relevant Information |  |

| 3. Summary of Engineering Assessment Methodology and Key Parameters Used |  |
| --- | --- |
| Occupancy Type(s) and Importance Level |  |
| Site Subsoil Class |  |
| For an ISA: |  |
| Summary of how Part B was applied, including: Key parameters such as μ, Sp and F factors Any supplementary specific calculations |  |
| For a DSA: |  |
| Summary of how Part C was applied, including: the analysis methodology(s) used from C2 other sections of Part C applied |  |
| Other Relevant Information |  |

| 4. Assessment Outcomes |  |  |
| --- | --- | --- |
| Assessment Status (Draft or Final) |  |  |
| Assessed %NBS Rating |  |  |
| Seismic Grade and Relative Risk (from Table A3.1) |  |  |
| For an ISA: |  |  |
| Describe the Potential Critical Structural Weaknesses |  |  |
| Does the result reflect the building’s expected behaviour, or is more information/ analysis required? | Yes – the ISA is sufficient Or No - a DSA is recommended |  |
| If the results of this ISA are being used for earthquake prone decision purposes, and elements rating <34%NBS have been identified: | Engineering Statement of Structural Weaknesses and Location | Mode of Failure and Physical Consequence Statement(s) |
| For a DSA: |  |  |
| Comment on the nature of Secondary Structural and Non-structural elements/ parts identified and assessed |  |  |
| Describe the Governing Critical Structural Weakness |  |  |
| If the results of this DSA are being used for earthquake prone decision purposes, and elements rating <34%NBS have been identified (including Parts): | Engineering Statement of Structural Weaknesses and Location | Mode of Failure and Physical Consequence Statement(s) |
| Recommendations (optional for EPB purposes) |  |  |

Sources of Information

Existing Documentation

The seismic assessment is based on the following:

<xx>

<xx>

Commentary:

Provide a list of all existing information available. Provide the author, date, revision number and page numbers for each source (where possible).

Geotechnical Information

Existing geotechnical information consists of the following:

<xx>

Commentary:

Provide a list of the geotechnical information available. Provide the author, date, revision number and page numbers for each source (where possible).

Site Observations and Investigations

A non-intrusive, visual site walk-over was completed by a structural engineer on <XX MONTH XXXX>. The purpose of the inspection was to verify the structural layout, where possible, against available structural drawing information.

<photo>

Photo 1: <description/comment>

<photo>

Photo 2: <description/comment>

Commentary:

List observations made and any intrusive investigations carried out. Provide a comment on the building condition relating to the structural elements.

Add rows to the table as required to ensure a good visual summary of the building is provided

Basis of Assessment

Analysis Methodology

The building was designed in <xxxx> by <designer (if known)>. It was designed to the then current building code <NZS xxxx (if known)> or the applicable design code for this time was < NZS xxxx (if known)>.

Commentary:

State the analysis approach used as the basis to determine the seismic capacity of the building (i.e. how the Engineering Assessment Guidelines were applied) including any software used. Comment on whether historical/original calculations were available to assist with the assessment. State what elements have been specifically reviewed.

Material Properties

The following table summarises the material proposed used.

Table 7: Material Properties

| Material | Assumed Probable Strength | Comment |
| --- | --- | --- |
| Concrete |  | <include source of information> |
| Reinforcing Steel High Yield |  |  |
| Reinforcing Steel Low Yield |  |  |
| Cold Drawn Mesh |  |  |
| Structural Steel | Plates and Sections: |  |
| Timber |  |  |
| Bolts |  |  |
| Weld Materials |  |  |

Commentary:

List the material properties utilised in the assessment and overstrength factors (if used). The list above is not exhaustive.

Describe what intrusive investigations were carried out and what was found. If “none” describe basis for assumed materials and structural systems.

Input detailed results and photos from any destructive testing, ferro scanning or investigative work.

Key Input Parameters

Seismic Loads

The following table summarises the principal parameters used for the derivation of earthquake loads and the analysis of the building.

Table 8: Seismic Loads

| Parameter | Value | Comments |
| --- | --- | --- |
| Design working life | <X years> |  |
| Importance level | <X> | <Make reference to Table 5.2 from the SGR> |
| Return period factor, R | <X> |  |
| Site subsoil classification | <X> |  |
| Period, T | <X.Xs> (longitudinal direction) <X.Xs> (transverse direction) | <state whether assumed or derived> |
| Hazard factor, Z | <X.XX> |  |
| Near fault factor, N(T,D) | <X.X> |  |
| Ductility factors,  | <X> | <stating whether the value is assumed or derived> |
| Structural performance factor, Sp | <X.X> |  |

Commentary:

Provide assumptions if necessary, and how these assumptions were derived and the background.

Dead and Live Loads

The following table summarises the dead and live loads for the structure.

Table 9: Dead and Live Loads

| Parameter | Value | Comments |
| --- | --- | --- |
| Dead loads | <from elements self-weights> |  |
| Superimposed dead loads |  |  |
| Live loads |  |  |

Assumptions

Commentary:

Provide a list of assumptions and how these assumptions were derived and the background.

Existing Documentation

Detailed Seismic Assessment Calculations and Analysis Results

This should include reference to the engineer’s Practice Field being in Structural Engineering, and commentary on experience in seismic assessment and recent relevant training

Or justification of assumptions if no drawings were able to be obtained

Indicate what form should the DSA take/ what the specific areas to focus on are

If a building comprises a shared structural form or shares structural elements with other adjacent titles, information about the extent to which the low scoring elements affect, or do not affect the structure.