# Routine Maintenance Plan

| Name of your school ¦ kura |  |
| --- | --- |
| Created by |  |
| Date last reviewed |  |

| Part of building | Task (see section 4 for details | Frequency | Who will do this | Date scheduled | Date completed |
| --- | --- | --- | --- | --- | --- |
| Roofs and roof flashings | Wash down | Annually or 6-monthly in seaspray and geothermal zones |  |  |  |
| Walls, windows, doors, window and door flashings | Wash down | Annually or 6-monthly in seaspray and geothermal zones |  |  |  |
| Gutters and downpipes | Clear and clean gutters | Annually (late autumn is ideal) |  |  |  |
| Plants close to buildings | Prune plants away from buildings (cladding and subfloor) and fans on heat pump units | Annually (late autumn is ideal) |  |  |  |
| Building exterior – all parts | Carry out preventive maintenance plan checks | Annually |  |  |  |
| Building exterior – all painted surfaces | Painting | Every 8–10 years |  |  |  |

# Preventative Maintenance Plan

| Name of your school ¦ kura |  |  |  |
| --- | --- | --- | --- |
| Created by |  | Scheduled inspection date |  |
| Inspected by |  | Date inspected |  |

| Part of building (and relevant section in this guide) | Look for these issues | Note details of all issues found | Is this work urgent? | Action/ repair required |
| --- | --- | --- | --- | --- |
| Walls (5.1) |  |  |  |  |
| Claddings generally (5.1.1) | Dirt, grime, salt, mould and lichen Paint fading, peeling, bubbling or cracked. Plant growth up or in wall Corrosion of fixings Nails sticking out from cladding |  |  |  |
| Concrete (5.1.2) | Cracking Corrosion of reinforcing steel Cracked or flaking concrete (spalling) Rust stains on concrete or concrete block walls Efflorescence or paint failure |  |  |  |
| Exterior insulation and finish systems (EIFS) (5.1.3) | Impact damage Coating failure or surface cracking Cladding in close contact with the ground Water entry and deterioration of parapet |  |  |  |
| Fibre-cement claddings (5.1.4) | Impact damage Damaged PVC joints or soakers Horizontal or vertical cracking Damaged or leaking control joints Swelling of sheet edges Cladding too close to ground level Leaking through cladding Paint fading or chalking |  |  |  |
| Masonry veneer systems (5.1.5) | Efflorescence Crumbling mortar Cracks in mortar, bricks or blocks Blocked or no ventilation or drainage slots Uneven colouring or staining |  |  |  |
| Metal claddings (5.1.6) | Dents, impact damage, buckling, tearing or scratches Corrosion or rust Paint peeling, bubbling, flaking or cracked Aluminium plank cladding buckling |  |  |  |
| Plywood claddings (5.1.7) | Sheets delaminating or cupping Paint peeling, flaking or faded Failure at sheet edges |  |  |  |
| Stonework (5.1.8) | Surface damage, deterioration or cracks Dampness in walls Crumbling mortar |  |  |  |
| Stucco (5.1.9) | Impact damage Surface cracking Touching or in the ground |  |  |  |
| Timber claddings (5.1.10) | Paint fading, peeling, bubbling, or cracked Joints opening Rotten, leaking, split or cracked weatherboards Rot along bottom of walls |  |  |  |
| Windows and doors (5.2) |  |  |  |  |
| Aluminium windows and doors (5.2.1) | Glazing seals failing Pitting of anodised finish Chalking of powder-coated finish Leaking at corner joints Sliding door sticking |  |  |  |
| Glazing (5.2.2) | Glass broken, stained, scratched or damaged Putty cracked or damaged Moisture in double-glazed units |  |  |  |
| Hardware for windows and doors (5.2.3) | Surface deterioration or corrosion Door handles loose, missing or not working Window hinges stiff or not working Self-closing doors not working |  |  |  |
| Steel windows and doors (5.2.4) | Rusted frames Leaking junctions Cracked or damaged putty |  |  |  |
| Timber doors and frames (5.2.5) | Difficult to close Finish deterioration Letting in water or draughts |  |  |  |
| Timber windows and frames (5.2.6) | Rotten window sill Casement sash won’t open Window draughts Timber is bowed Signs of water ingress Hardware is loose or corroded |  |  |  |
| Roofs (5.3) |  |  |  |  |
| Profiled metal roofing (5.3.1) | Buckling of roofing, ridging or flashings Corrosion at galvanised steel sheet ends Dented or damaged sheets Roofing nails damaged or missing Neoprene washers missing from screws Lifting of roof or flashing Ponding water Red rusting of galvanised steel Surface pitting of aluminium White rusting of galvanised steel |  |  |  |
| Coil-coated steel (5.3.2) | Chalking or fading of paint finish Corroded roofing, flashings or fixings Paint damage or scratches |  |  |  |
| Membrane roofing (liquid-applied or paint-on) (5.3.3) | Bubbles appearing under the membrane Chalking Popped nails in the substrate Surface of membrane buckling or cracking Water lying on the surface |  |  |  |
| Membrane roofing (sheet applied) (5.3.4) | Bubbles appearing under the membrane Damage to waterproof material Lifting of sheet joints Nail popping in timber substrates Roof feels springy or spongy Splitting/surface cracking in the membrane Water ponding on the surface |  |  |  |
| Tiles – metal (5.3.5) | Corrosion Dented or damaged tiles Loss of stone chips Moss and lichen growth Tile edge lifting Water not draining to gutter |  |  |  |
| Tiles – clay and concrete (5.3.6) | Dirt, efflorescence, moss or lichen Corroded valley gutters Poor visual appearance Broken, cracked, dislodged, loose or missing tiles Lead flashing lifting Roof sagging |  |  |  |
| Translucent sheeting (5.3.7) | Cracked sheets Steel mesh missing or damaged Damaged fixing seals Dirt trapped in the sheets Impact damage Loose sheets Discolouration Surface deterioration or crazing |  |  |  |
| Roof penetrations (5.3.8) | Chimney or flue flashing lifted Corrosion of metal flashings Insecure flues or vent pipes Safety feature connections unsafe Poor condition of sealant and seals |  |  |  |
| Roof glazing and skylights (5.3.9) | Dirty roof glazing Aluminium frames pitting Cracked roof glazing Moisture getting into double- glazed units Ponding around roof penetrations or skylights. |  |  |  |
| Spouting and gutters (5.3.10) | Dirt, mould or lichen growth Corroded/deformed metal gutters Corroded metal gutters Deformed metal gutters Water spilling from external gutter Overflowing internal gutters Metal gutter joint failure PVC gutters damaged or cracked Water runs down fascia |  |  |  |
| Downpipes (5.3.11) | Dirt, moss or lichen Impact damage Blocked stormwater drain Corrosion of metal downpipes Downpipe not connected to gutter Downpipe not discharging into drain |  |  |  |
| Subfloors (5.4) |  |  |  |  |
| Concrete foundation and basement walls (5.4.1) | Signs of moisture below ground level Cracked concrete or cement plaster Subsided or uneven floor levels Damaged timber-framed subfloor foundation walls |  |  |  |
| Electrical wiring (5.4.2) | Damaged wiring Loose wiring |  |  |  |
| Insulation (5.4.3) | No underfloor insulation Poorly installed insulation |  |  |  |
| Subfloor moisture (5.4.4) | Damp ground conditions Damp stains on underside of flooring Mould and water droplets under flooring Ponding water Musty smells Timber decay Moisture within subfloor areas |  |  |  |
| Timber-framed foundations (5.4.5) | Bearers or joists damaged Bolted connections loose Borer in joists, bearers, bottom plates Gap between pile and bearer Lateral bracing missing or damaged Rotted or undermined timber piles Rusted fixings |  |  |  |